# SEC-PROC-PATCH-001 V2 - Plant Patch and Vulnerability Management Standard (APPROVED)

Synthetic training document. Not for operational use.

Document ID: SEC-PROC-PATCH-001 | Version 2.0 | Status: APPROVED | Effective: 2025-06-01 | Owner: Plant Cyber Security | Approver: Head of Engineering and CISO delegate

## 1. Purpose

Defines remediation targets and handling rules for vulnerabilities on plant assets, condition-monitoring gateways and engineering workstations.

## 2. Severity and remediation targets

Severity follows the CVSS-style base score recorded on the vulnerability register.

| Severity | Score range | Remediate within |
| --- | --- | --- |
| Critical | 9.0 - 10.0 | 7 days |
| High | 7.0 - 8.9 | 14 days |
| Medium | 4.0 - 6.9 | 30 days |
| Low | 0.1 - 3.9 | 90 days |

The clock starts on the found date. An open item past its target is OVERDUE and must be escalated.

## 3. Operational technology rules

- A patch on a plant-connected asset is never applied during operation. It needs an approved maintenance window and a sign-off from the responsible engineering owner.

- If a patch cannot be applied inside the target, a compensating control (for example network isolation or disabling the affected service) must be recorded and the item moves to ACCEPTED RISK only with written approval from the Head of Engineering.

- Condition-monitoring gateways that are Restricted or External exposure are reviewed first.

## 4. Escalation

Overdue High or Critical items on an asset with Critical or High criticality are reported to the Plant Cyber Security lead within 2 working days, together with the current operational health status of the asset so engineering can plan the window.

## 5. Limits

This standard does not authorise anyone to apply a patch, change a configuration or close a security item. A qualified human owner decides and records the outcome.