# SEC-STD-ZONES-001 - OT Network Segmentation and Zone Standard (APPROVED)

Synthetic training document. Not for operational use.

Document ID: SEC-STD-ZONES-001 | Version 1.1 | Status: APPROVED | Effective: 2025-10-01 | Owner: Plant Cyber Security | Approver: Head of Engineering

## 1. Purpose

Defines the trust zones for plant assets and the rules for traffic between them. The zone list is held in the security_network_zones table.

## 2. Zones

| Zone | Description | Level |
| --- | --- | --- |
| Z-L0 | Field devices: sensors and actuators | Level 0 |
| Z-L1 | Basic control: controllers and condition-monitoring gateways | Level 1 |
| Z-L2 | Supervisory: HMI and SCADA | Level 2 |
| Z-L3 | Operations: historian and engineering workstations | Level 3 |
| Z-DMZ | Industrial DMZ: remote access and data transfer | Level 3.5 |
| Z-L4 | Enterprise IT | Level 4-5 |

## 3. Exposure classes

- Internal: reachable only from inside the plant network.

- Restricted: reachable from outside through a controlled, logged remote access path in the DMZ.

- External: reachable from outside the plant through the DMZ with a service published to the corporate or vendor network.

An asset moves from Internal to Restricted or External only with a written exception approved by the Head of Engineering.

## 4. Traffic rules

- No direct traffic from Z-L4 or from outside the plant to Z-L1 or Z-L0. It must pass through the DMZ.

- Remote sessions end in the DMZ and are brokered into Z-L3. They are time-boxed and recorded.

- Outbound transfers from Z-L1 to external addresses are denied by default. Backups go to an internal backup server only.

- Unapproved scanning is prohibited. Approved scans run from the registered scanner address under a change ticket.

## 5. Review

Zone membership and exposure class are reviewed every six months and after any incident that involved lateral movement.